On-Site Live HV Work Technique Review (HV Stick)

Team Members………………………………………………………………………………………………………………

Job Location…………………………………………………………………………………………………………………

Task……………………………………………………………………………………………………………………………

Date……/………/…….

| No. | Item | Compliance |  |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  |  | Yes | No | N/A |  |
|  | Undertake Hazard & Risk assessment - Voltage identification - Structure security/soundness inc adjacent poles - Manage step & Touch potential - Communication link to control room available - Live Line protection enabled - Traffic and pedestrian control - Safe to approach test (SWER) |  |  |  |  |
|  | Use of Insulated Sticks - Clean/prepare for use - Inspect sticks and brackets for problems. - Check within test. - Setting of MAD markers. - Check operation of tools attached - Check tool operation (shotgun sticks) - Correct tools/sticks selected - Tool cord replaced prior to days work |  |  |  | Next Test Date: / / |
|  | Act as a Safety Observer - Concentration maintained - Effective Communication with crew aloft - Positioned to view personnel aloft - Vest/identification worn. - Verbalised warning for crew aloft if applicable |  |  |  |  |
| No. | Task/Technique | Compliance |  |  | Comments |
|  |  | Yes | No | N/A |  |
|  | Use EWP - Prestart checks completed. - Within Electrical Test - Secondary EWP controls working - Escape devices present - Ground person aware of rescue methods - EWP chassis Earthed - Clearances from uninsulated parts maintained. - Insulated section cleaned and inspected. - Clearance from lower circuit managed. - Drop zone covered if clearance can’t be maintained. - SWL not exceeded - Hydraulic tool hose condition / test date |  |  |  | Next Test Date: / / Next Test Date: / / |
|  | Use Fall Prevention Equipment. - Inspected prior to use. - Correct adjustment of harness - Lanyard attached with energy absorber end to fall arrest point of harness. - Dry lanyards used. - Within inspection date |  |  |  | Next Inspection Date: / / |
|  | Live Line rope management. - Inspection of rope for problems. - Correct knotting used. - Insulators used in taglines/headropes. - Dry ropes used. - Ropes tested where applicable |  |  |  |  |
|  | Working with Cranes - Correct signals or standard terminology used. - One person direction of Crane operator. - Clear directions conveyed between persons aloft and ground man. |  |  |  |  |

| No. | Task/Technique | Compliance |  |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  |  | Yes | No | N/A |  |
|  | Make /Break bridges - Load switching device used correctly - Current loading identified /measured-checked for removal of load. - Load break components checked prior to installation - Jumper movement controlled/considered. - Connections cleaned before jointing. - Correct connectors used &installed in correct orientation. - 1.6km distance not exceeded |  |  |  |  |
|  | Minimum Approach Distances - Body positioning for task adequate - EWP/Platform placement adequate - Mobile plant used not breaking MAD |  |  |  |  |
|  | Crossarm Replacement - Intermediate - Angle - Termination - Correct helical terminations selected - Load change considered on adjacent structures |  |  |  |  |

## General Comments: